Title: RTL Design of Finite State Machines (FSM) and Control Units

Intern Name: Lagisetty Vidya Charan

College: Mohan Babu University

Department: Electronics and Communication Engineering

Internship: VLSI Design Internship

Tool Used: Xilinx Vivado

HDL: Verilog HDL

Simulation: Vivado Behavioral Simulation

- Introduction:

A Finite State Machine (FSM) is a sequential digital system that operates through a finite number of predefined states. The FSM changes from one state to another depending on the current state, input conditions and clock signal.

FSMs are widely used in digital systems for implementing control logic. They are commonly used in processors, communication protocols, embedded systems and ASIC/FPGA controllers. The Task 4 document specifically introduces FSMs as an important part of control-driven digital design.

An FSM generally consists of:

- States

- Inputs

- Outputs

- State transitions

There are two major types of FSM:

Moore FSM

In a moore machine, the output depends only on the current state.

Output=f(State)

Mealy FSM

In a mealy machine, the output depends on both the current state and input

Output=f(State, Input)

These definitions and concepts are part of the FSM fundamentals specified in the task.

- Objectives:

The objectives of this task are:

- To understand the fundamentals of Finite State Machines.

- To understand the difference between Moore and Mealy FSMs.

- To design FSMs using Verilog HDL.

- To implement a traffic light controller using an FSM.

- To design a binary sequence detector for the pattern 1011.

- To develop testbenches for the RTL designs.

- To perform behavioral simulation using Xilinx Vivado.

- To analyze state transitions and output behavior using simulation waveforms.

- Software And Tools Used:

Xilinx Vivado

Xilinx Vivado was used for:

- Creating the RTL project

- Writing Verilog modules

- Creating simulation testbenches

- Running behavioral simulations

- Viewing simulation waveforms

- Verifying state transitions and outputs

- MOORE FSM

  - Problem Statement

Design a simple FSM that continuously cycles through three states:

S0 → S1 → S2 → S0

The state encoding used is:

| State | Binary |
| --- | --- |
| S0 | 00 |
| S1 | 01 |
| S2 | 10 |

This is the Moore FSM example

  - Working Principle

The FSM uses a clock and reset signal.

When reset is asserted, the FSM enters state S0.

After reset is released, the FSM changes state on every positive edge of the clock.

The transition sequence is:

S0 → S1

S1 → S2

S2 → S0

The state is stored in a 2-bit register.

  - Verilog Implementation

module moore_fsm(

input clk,

input reset,

output reg [1:0] state

);

parameter S0 = 2'b00,

S1 = 2'b01,

S2 = 2'b10;

always @(posedge clk or posedge reset) begin

if (reset)

state <= S0;

else begin

case (state)

S0: state <= S1;

S1: state <= S2;

S2: state <= S0;

default: state <= S0;

endcase

end

end

endmodule

  - Testbench

The testbench generates a clock and applies reset to the FSM.

module tb_moore_fsm;

reg clk;

reg reset;

wire [1:0] state;

moore_fsm uut (

.clk(clk),

.reset(reset),

.state(state)

);

initial clk = 0;

always #5 clk = ~clk;

initial begin

reset = 1;

#10;

reset = 0;

#50;

$finish;

end

endmodule

  - Simulation Result

Observation:

The simulation waveform shows that the FSM starts from S0 after reset and continuously cycles through:

00 → 01 → 10 → 00

Therefore, the required Moore FSM state transitions were successfully verified.

- Mealy FSM

  - Problem Statement

The Mealy FSM is designed such that the output becomes HIGH when:

Current State = S1

AND

Input = 1

  - State Transition Table

| Current state | Input | Next state | Output |
| --- | --- | --- | --- |
| S0 | 0 | S0 | 0 |
| S0 | 1 | S1 | 0 |
| S1 | 0 | S0 | 0 |
| S1 | 1 | S0 | 1 |

The important condition is:

S1 + Input 1 → Output 1

  - Working Principle

Initially, the FSM is in S0.

When input becomes 1, the FSM transitions from S0 to S1.

If the next input is also 1, the FSM produces:

out=1

because the current state is S1 and the input is 1.

This demonstrates that the output depends on both state and input.

  - Verilog Implementation

`timescale 1ns / 1ps

module mealy_fsm(

input clk,

input reset,

input in,

output reg out

);

reg state;

parameter S0 = 1'b0,

S1 = 1'b1;

always @(posedge clk or posedge reset) begin

if (reset) begin

state <= S0;

out <= 1'b0;

end

else begin

case (state)

S0: begin

if (in) begin

state <= S1;

out <= 1'b0;

end

else begin

state <= S0;

out <= 1'b0;

end

end

S1: begin

if (in) begin

out <= 1'b1;

state <= S0;

end

else begin

out <= 1'b0;

state <= S0;

end

end

default: begin

state <= S0;

out <= 1'b0;

end

endcase

end

end

endmodule

  - Simulation Result

Observation: The simulation verifies that the output becomes HIGH when the FSM is in S1 and the input is 1.

This demonstrates the fundamental difference between Moore and Mealy machines: the Mealy output is influenced by the input as well as the current state.

- Traffic Light Controller

  - Problem Statement

A traffic light controller is implemented using an FSM.

The task defines:

| State | Light |
| --- | --- |
| S0 | RED |
| S1 | YELLOW |
| S2 | GREEN |

The light encodings are:

RED    = 100

YELLOW = 010

GREEN = 001

These values are given in the task’s starter design.

  - Working Principle

The controller uses a 2-bit state register.

After reset:

State=S0

Light=RED

The controller then progresses through the defined states on every positive clock edge.

The implemented output sequence observed in simulation is:

RED → GREEN → YELLOW → RED

The corresponding binary values are:

100 → 001 → 010 → 100

  - Verilog Implementation

`timescale 1ns / 1ps

module traffic_light(

input clk,

input reset,

output reg [2:0] light

);

parameter RED    = 3'b100,

YELLOW = 3'b010,

GREEN  = 3'b001;

reg [1:0] state;

parameter S0 = 2'b00,

S1 = 2'b01,

S2 = 2'b10;

always @(posedge clk or posedge reset) begin

if (reset) begin

state <= S0;

light <= RED;

end

else begin

case (state)

S0: begin

state <= S1;

light <= GREEN;

end

S1: begin

state <= S2;

light <= YELLOW;

end

S2: begin

state <= S0;

light <= RED;

end

default: begin

state <= S0;

light <= RED;

end

endcase

end

end

endmodule

  - Simulation Result

Observation: The waveform confirms that the controller changes its light output according to the FSM state and that the sequence repeats continuously after reset.

- 1011 Sequence Detector

  - Problem Statement

The objective is to detect the binary sequence:

1011

from a serial input stream.

  - State Definition

Four states are used.

| State | Meaning | Encoding |
| --- | --- | --- |
| S0 | No bits matched | 000 |
| S1 | 1 detected | 001 |
| S2 | 10 detected | 010 |
| S3 | 101 detected | 011 |

  - Working Principle

The FSM remembers how many bits of the required pattern have already been received.

Step 1

Input: 1

The FSM moves: S0 → S1

Now one bit of the pattern has been matched.

Step 2

Input: 0

The FSM moves: S1 → S2

The matched pattern is now: 10

Step 3

Input: 1

The FSM moves: S2 → S3

The matched pattern is: 101

Step 4

Input: 1

The complete pattern becomes: 1011

Therefore: detected=1

  - Verilog Implementation

`timescale 1ns / 1ps

module sequence_detector(

input clk,

input reset,

input in,

output reg detected

);

reg [2:0] state;

parameter S0 = 3'b000,

S1 = 3'b001,

S2 = 3'b010,

S3 = 3'b011;

always @(posedge clk or posedge reset) begin

if (reset) begin

state <= S0;

detected <= 1'b0;

end

else begin

case (state)

// No matching bits

S0: begin

detected <= 1'b0;

if (in)

state <= S1;

else

state <= S0;

end

// Detected "1"

S1: begin

detected <= 1'b0;

if (in)

state <= S1;

else

state <= S2;

end

// Detected "10"

S2: begin

detected <= 1'b0;

if (in)

state <= S3;

else

state <= S0;

end

// Detected "101"

S3: begin

if (in) begin

detected <= 1'b1;

state <= S1;

end

else begin

detected <= 1'b0;

state <= S2;

end

end

default: begin

state <= S0;

detected <= 1'b0;

end

endcase

end

end

endmodule

  - Simulation Result

The important waveform relationship is:

Input: 1 → 0 → 1 → 1

State: S0 → S1 → S2 → S3

Detection: 0 → 0 → 0 → 1

Observation: The simulation successfully detects the 1011 sequence. The detected signal becomes HIGH after the final 1 is received.

The implementation also returns to S1 after detection, allowing another sequence beginning with 1 to be recognized.

- Comparison of Moore and Mealy FSM

| Feature | Moore FSM | Mealy FSM |
| --- | --- | --- |
| Output depends on | State | State + Input |
| Output timing | State dependent | Can respond to input |
| Design used | 3-state counter like FSM | Input dependent output FSM |
| Main concept demonstrated | State transition | State + input control |
| Output behavior | Determined by current state | Determined by current state and input |

- Simulation And Verification

All four designs were verified using Vivado Behavioral Simulation.

The verification process consisted of:

1. Creating the RTL module.

2. Creating a corresponding Verilog testbench.

3. Generating a clock signal.

4. Applying reset.

5. Applying required input patterns where applicable.

6. Running behavioral simulation.

7. Observing state transitions.

8. Observing output signals.

9. Comparing the waveform against the expected behavior.

The task specifically requires waveform analysis to verify state transitions, output timing and reset behavior.

- Results

The following results were obtained:

| Design | Result |
| --- | --- |
| Moore FSM | Successfully cycled through S0 → S1 → S2 |
| Mealy FSM | Output HIGH for S1 with input=1 |
| Traffic Light Controller | Successfully generated RED → GREEN → YELLOW sequence |
| 1011 Sequence Detector | Successfully detected 1011 |

Therefore, all four RTL designs were successfully simulated and verified.

- Learning Outcomes

Through this task, the following concepts were understood:

- Finite State Machine architecture

- Moore FSM

- Mealy FSM

- State encoding

- State transition logic

- Sequential Verilog design

- always @(posedge clk or posedge reset)

- case-based FSM implementation

- Testbench development

- Behavioral simulation

- Waveform analysis

- Traffic-light control using FSM

- Serial pattern detection

- Overlapping sequence detection

- Conclusion

The Task 4 objective of implementing FSM-based RTL designs was successfully achieved using Verilog HDL and Xilinx Vivado.

Four different FSM applications were implemented: a Moore FSM, a Mealy FSM, a traffic light controller and a 1011 sequence detector. Each design was tested using a dedicated Verilog testbench and verified through behavioral simulation waveforms.

The simulations demonstrated correct state transitions, reset behavior and output generation. The task also provided practical understanding of how FSMs are used to implement control logic in digital systems.