Title: Sequential Circuit Design and Simulation

Intern Name: Lagisetty Vidya Charan

College: Mohan Babu University

Department: Electronics and Communication Engineering

Internship: VLSI Design Internship

- Introduction:

Sequential circuits are digital circuits whose outputs depend on the current inputs as well as the previous state of the circuit. Unlike combinational circuits, sequential circuits use storage elements and are generally controlled by a clock signal.

In this task, basic sequential circuits were designed and simulated using Verilog HDL and Xilinx Vivado. The implemented circuits include a D Flip-Flop, JK Flip-Flop, 4-bit Register, and 4-bit Binary Counter. Each design was verified through behavioral simulation using an appropriate Verilog testbench.

- Objective:

The objectives of this task are:

- To understand the basic operation of sequential circuits.

- To implement a D Flip-Flop using Verilog HDL.

- To implement a JK Flip-Flop and verify its different operating modes.

- To design a 4-bit register for data storage.

- To design a 4-bit binary counter.

- To create testbenches for each circuit.

- To verify the designs through behavioral simulation in Vivado.

- To analyze and document the resulting simulation waveforms.

- Tools Used:

| Tool | Purpose |
| --- | --- |
| Verilog HDL | Hardware description and RTL implementation |
| Xilinx Vivado | Design entry and simulation |
| Behavioral Simulation | Functional verification |
| GitHub | Source-code and project documentation |

- D Flip-Flop:

4.1 Description

A D Flip-Flop is a sequential storage element that captures the value present at its D input at the rising edge of the clock. The captured value is then maintained at the output until the next active clock edge.

The implementation uses a positive-edge-triggered clock.

  - Verilog Implementation

module d_flipflop(

input clk,

input D,

output reg Q

);

always @(posedge clk)

begin

Q <= D;

end

endmodule

  - Testbench

The testbench generates a periodic clock and applies different values to the D input to verify that Q captures D at the rising clock edge.

  - Simulation Result

Figure 1: Behavioral simulation waveform of D Flip-Flop

  - Observation

The simulation verifies that the D Flip-Flop captures the input D at each rising edge of the clock. The output Q changes according to the value of D at the active clock edge and remains stored between clock edges.

The initial X observed in Q before the first valid clock operation is due to the absence of an initial/reset value.

- JK Flip-Flop:

5.1 Description

A JK Flip-Flop is a clock-controlled sequential circuit with two inputs, J and K. It provides four operating modes depending on the values of J and K. The task specifies the following operations.

| J | K | Operation |
| --- | --- | --- |
| 0 | 0 | No Change |
| 0 | 1 | Reset |
| 1 | 0 | Set |
| 1 | 1 | Toggle |

  - Verilog Implementation

module jk_flipflop(

input clk,

input J,

input K,

output reg Q

);

always @(posedge clk)

begin

case ({J,K})

2'b00: Q <= Q;

2'b01: Q <= 1'b0;

2'b10: Q <= 1'b1;

2'b11: Q <= ~Q;

endcase

end

endmodule

  - Testbench

The testbench applies all four combinations of J and K:

00 → No Change

10 → Set

01 → Reset

11 → Toggle

This allows all operating modes to be verified.

  - Simulation Result

Figure 2: Behavioral simulation waveform of JK Flip-Flop

  - Observation

The simulation successfully verifies all four operating modes of the JK Flip-Flop. The output remains unchanged for J=0, K=0, becomes set for J=1, K=0, resets for J=0, K=1, and toggles for J=1, K=1.

- 4-bit Register:

6.1 Description

A register is a group of storage elements used to store binary data. In this task, a 4-bit register is implemented to store a 4-bit input value at the rising edge of the clock.

  - Verilog Implementation

module register4(

input clk,

input [3:0] D,

output reg [3:0] Q

);

always @(posedge clk)

begin

Q <= D;

end

endmodule

  - Testbench

The testbench applies the following 4-bit data values:

0000

1010

0101

1111

0011

The register captures each value on the rising edge of the clock.

  - Simulation Result

Figure 3: Behavioral simulation waveform of 4-bit Register

  - Observation

The simulation confirms that the 4-bit register successfully stores the input data at the rising edge of the clock.

The following values were successfully captured:

0000 → 1010 → 0101 → 1111 → 0011

The output remains stable between active clock edges

- 4-bit Binary Counter:

7.1 Description

A binary counter is a sequential circuit that changes its state according to the clock signal. The implemented 4-bit counter increments its value by one at every rising edge of the clock when reset is inactive.

A synchronous reset is included to initialize the counter to zero.

  - Verilog Implementation

module counter4(

input clk,

input reset,

output reg [3:0] count

);

always @(posedge clk)

begin

if (reset)

count <= 4'b0000;

else

count <= count + 1'b1;

end

endmodule

  - Testbench

Initially, reset is asserted to initialize the counter:

reset = 1

count = 0000

After reset is released, the counter increments on every rising edge

  - Simulation Result

Figure 4: Behavioral simulation waveform of 4-bit Binary Counter

  - Observation

The simulation verifies the correct counting sequence:

0000

0001

0010

0011

0100

0101

0110

0111

1000

Thus, the counter successfully increments by one at each rising edge of the clock after reset is released. The task specifies this incrementing behavior for the binary counter.

- Overall Simulation Results:

| Circuit | Inputs Tested | Simulation Result |
| --- | --- | --- |
| D Flip-Flop | D and CLK | Verified |
| JK Flip-Flop | J, K, CLK | All four modes verified |
| 4-bit Register | 4-bit D and CLK | Data captured correctly |
| 4-bit Counter | CLK and RESET | Incrementing sequence verified |

All four sequential circuits were successfully implemented in Verilog HDL and verified using behavioral simulation in Vivado.

- Conclusion:

The task successfully demonstrated the design and simulation of fundamental sequential circuits using Verilog HDL. A D Flip-Flop was implemented to demonstrate single-bit data storage, while the JK Flip-Flop was verified for its four operating modes. A 4-bit register was designed to store multi-bit data, and a 4-bit binary counter was implemented to increment its state on successive clock cycles.

The behavioral simulation waveforms confirmed that all four designs operate according to their intended functionality. This task provided practical experience in designing clock-driven sequential circuits, writing Verilog testbenches, running simulations, and analyzing waveform outputs.